AI-Based Multi-Sensor Smart Wearable for Non-Invasive Health and Athletic Monitoring

Raghunandhan, 1 Thilak V N, 1 UdayKrishna, 1 Krishnaraman, 1 Dr. Anu G. Kumar1,∗

1Department of Electrical and Electronics Engineering,

Amrita School of Engineering, Amrita Vishwa Vidyapeetham, Coimbatore, India

∗gk_anu@cb.amrita.edu

Abstract— This project focuses on designing a smart, AI-based wearable system that continuously monitors glucose trends along with multiple physiological and athletic parameters using non-invasive sensors. The system combines multi-sensor data with edge artificial intelligence to provide real-time health and performance insights directly on the device, eliminating the need for invasive tests or cloud-based processing.

- Introduction

The global prevalence of Type 2 diabetes and metabolic health disorders has reached epidemic levels, with the International Diabetes Federation estimating over 537 million affected adults worldwide as of 2021 — a figure projected to exceed 780 million by 2045 [1]. A critical barrier to early intervention is the continued dependence on invasive blood-based glucose monitoring, which is painful, impractical for frequent use, and inaccessible in resource-limited settings.

As of 2021, the International Diabetes Federation estimated that over 537 million adults worldwide had Type 2 diabetes and metabolic health disorders; by 2045, that number is expected to rise to over 780 million [1]. The incidence of these conditions has reached epidemic proportions. The ongoing reliance on intrusive blood-based glucose monitoring, which is uncomfortable, inconvenient for regular usage, and unavailable in settings with low resources, is a major obstacle to early intervention.

- Objective

This project's main goal is to create a non-invasive, multi-sensor wearable gadget that can track body temperature, physical activity, weariness, and posture while also measuring glucose trends. In contrast to traditional monitoring systems [3], this platform enables edge-based inference without the need for external computing infrastructure by simply deploying a trained AI model on a microcontroller.

- Motivation

Due to the intrusiveness and expense of current technologies, most people are still unable to achieve continuous metabolic monitoring in daily situations. Intelligent inference on ultra-low-power microcontrollers is now possible thanks to deep learning and TinyML frameworks [4], creating a feasible route for non-invasive, continuous health monitoring. The key goal is to minimise reliance on cloud connectivity while preserving model accuracy, especially for implementation in places with poor internet connectivity.

- Literature Survey

Previous studies have investigated the use of near-infrared spectroscopy for non-invasive glucose determination [5], showing a relationship between blood glucose levels and NIR absorption characteristics. Quantised neural network models can function within the memory and power limitations of ARM Cortex-M class processors, according to concurrent work on TinyML deployment for wearable health applications [6]. On low-power devices, edge-based IMU analysis for tiredness detection and activity classification has also been verified [7]. Nevertheless, no previous system has combined edge AI inference, multi-axis motion analysis, and glucose trend estimates into a single wearable device.

- Research Gap

  - Sensor Fusion for Metabolic Monitoring: The majority of wearable technology now in use treats glucose estimates and activity monitoring as distinct issues that are managed by different systems or devices. There is currently no evidence of a unified, fused multi-sensor platform for concurrent monitoring at the edge.

  - Cloud-Free AI Inference: Current AI-powered health wearables are not suitable for offline use and expose user data to third-party servers since they rely on cloud servers for model inference. By implementing a fully quantised TinyML model directly on the Arduino Nano 33 BLE Sense Rev2, this project fills the gap and allows offline, real-time inference without relying on the cloud.

II. Dataset and Label Description

This work's training and validation datasets were all self-collected using the wearable platform prototype. Our dataset was obtained through controlled physical activity sessions intended to simultaneously produce quantifiable changes in glucose trends, motion patterns, and heat response, in contrast to publicly accessible clinical datasets that collect static patient records.

A. Protocol for Gathering Data
A healthy adult participant was chosen for many days' worth of data gathering sessions. Every session included a predetermined series of physical states: jogging, high-intensity running, brisk walking, and resting (baseline). Each state was maintained for a certain amount of time—five minutes—with three-minute rest intervals in between.

- System Requirements

Hardware Requirements

| Component | Specification | Role |
| --- | --- | --- |
| Arduino Nano 33 BLE Sense Rev2 | ARM Cortex-M4F, 64 MHz, 1 MB Flash | Main processor and inference engine |
| NIR Sensor (External) | Near-Infrared Optical Module | Glucose trend estimation from wrist |
| 9-Axis IMU (Built-in) | Accelerometer, Gyroscope, Magnetometer | Activity, posture, fatigue detection |
| Temperature Sensor (Built-in) | Onboard thermal sensor | Skin temperature and exertion response |
| Microphone (Built-in) | Digital MEMS | Breathing patterns (optional) |
| BLE 5.0 (Built-in) | Bluetooth Low Energy | Wireless output to phone or laptop |
| OLED Display | 0.96", 128×64 px | Real-time on-device readout |
| Li-Po Battery | 3.7 V, 300 mAh | Portable power |

Software Requirements

| Tool | Purpose |
| --- | --- |
| Python 3.x | Data preprocessing and model training |
| TensorFlow / Scikit-learn | Model development and evaluation |
| TensorFlow Lite for Microcontrollers | On-device TinyML deployment |
| Arduino IDE | Firmware development |
| ArduinoBLE Library | BLE communication |
| Arduino Sensor Libraries | Sensor interfacing |

- Collection Setup

The NIR sensor was positioned against the inner wrist to maintain consistent optical contact with the skin. A standard fingertip glucometer was used at the start and end of each session to capture reference blood glucose values, which were used to label the data with ground-truth glucose trend classes. All sessions were conducted indoors at consistent ambient conditions to reduce environmental noise in the optical signal.

- Session Protocol

Each session followed a fixed structure. The subject would begin seated at rest for five minutes, then move through brisk walking, jogging, and high-intensity running, each held for five minutes with short rest intervals in between. The entire session was repeated across three different dietary conditions: fasting in the morning before any food, one hour after a standard meal, and thirty minutes after completing a high-intensity run. Repeating across these conditions was important because glucose behaves quite differently depending on when and what you last ate, and we needed the model to learn those differences.

- Methodology

- Preprocessing and Feature Extraction

Once all session data was logged, it was brought into Python for processing. The raw sensor streams were segmented into two-second windows with 50% overlap between consecutive windows — so at 50 Hz, each window contained 100 samples. This window size was chosen because it is long enough to capture a full stride cycle and meaningful NIR fluctuation, but short enough to keep inference latency practical.

From each IMU window we extracted RMS acceleration, peak acceleration, mean and standard deviation of gyroscope readings, step frequency, jerk magnitude, and postural tilt angle. From each NIR window we extracted mean signal intensity, variance, slope, and differential absorption across the window. From the temperature stream we included mean skin temperature and rate of change. Each window ultimately produced a 24-dimensional feature vector, which became the input to the model.

- Model Architecture and Training

We kept the model deliberately small. The architecture is a feedforward neural network with 24 input neurons, two hidden layers of 32 and 16 neurons using ReLU activations, and a softmax output layer. Dropout regularization at 0.3 was applied after each hidden layer to guard against overfitting on our relatively modest dataset.

Training used an 80/20 stratified split, the Adam optimizer at a learning rate of 0.001, and ran for 80 epochs. The model reached 89.4% overall test accuracy. After training, it was converted to TensorFlow Lite and quantized to 8-bit integer precision, shrinking it from 43 KB down to 11 KB — well within the Arduino's 1 MB flash.

- On-Device Inference

On the device, the firmware runs a continuous loop. Sensors are sampled at 50 Hz into a circular buffer. When a full two-second window is ready, the feature extraction routine computes the same 24 features that were used during training and passes them to the TinyML interpreter. Inference takes between 14 and 22 milliseconds. The output class is decomposed into its activity and glucose trend components and sent to both the OLED display and the BLE transmitter.

A simple confidence filter suppresses any prediction below 0.55 softmax probability, holding the previous valid result instead. This prevents the display from flickering during momentary motion transitions or sensor noise, which made a noticeable practical difference during  testing.

- Limitations

The most significant limitation is that all data came from a single subject. The model has learned that person's physiology well, but it has not been tested on others. Skin tone, wrist geometry, body fat percentage, and individual metabolic responses all affect NIR signals and will introduce variability that the current model has not encountered. We treat this as a personalized health estimator for now, not a clinical diagnostic too.

- Results

The system was tested on the subject across multiple live sessions covering all four activity states — resting, walking, jogging, and running — under both fasting and post-meal conditions. The focus during testing was not just on classification accuracy numbers but on whether the device behaved usefully in practice: did it respond correctly when the subject shifted activity, did it catch fatigue when it set in, and did the glucose trend output shift in the expected direction after a meal. On all three fronts the system performed well enough to be considered functional for its intended purpose.

- Activity and Fatigue Detection

Activity classification worked reliably across the clearly separated states. The device correctly identified resting, walking, jogging, and running in the large majority of cases, with most of the confusion occurring at the boundary between walking and jogging — which is expected, since those two states blend into each other gradually rather than switching sharply.

The more practically useful output, and the one that drew the most attention during testing, was fatigue detection. As the subject continued running past a certain point, the IMU data began showing characteristic signs of physical fatigue — stride became less consistent, jerk magnitude increased, and cadence started to drop even though speed was being maintained. When these patterns crossed the threshold defined in the post-inference decision layer, the device triggered a fatigue alert on the OLED display.

The alert is straightforward — the display switches from showing the normal activity status to a visible warning indicating that fatigue has been detected and that the user should consider slowing down or stopping. The BLE output simultaneously transmits the fatigue flag to the paired device so it can be logged with a timestamp. During testing, this alert consistently appeared at a point in the run that the subject themselves described as the moment they started feeling the effort. That agreement between what the sensors detected and what the person actually felt is the most meaningful validation we could ask for from a single-subject test.

[Insert Fig. 1 — OLED display showing fatigue alert triggered during running session]

- Glucose Trend Output

The glucose trend output behaved as expected across the different metabolic conditions. During fasting sessions the device consistently reported a Normal trend status. During post-meal sessions the output shifted toward Elevated, particularly in the window between thirty and ninety minutes after eating, which aligns with the natural post-meal glucose rise. The system did not produce High-Risk readings during normal testing, which is expected since the subject is healthy, but the label is present in the model and is reserved for edge cases or future multi-subject testing.

The NIR signal showed clear and repeatable differences between fasting and post-meal conditions even when the subject was completely at rest, which is what gives the glucose trend output its basis. These differences became less distinct during high-intensity running, likely because increased blood perfusion and movement artifact both affect the optical signal simultaneously. The model handles this by weighting the IMU activity context alongside the NIR features, so the glucose output during running is interpreted differently than it would be at rest.

[Insert Fig. 2 — Glucose trend output across fasting, post-meal, and running conditions]

- On-Device Performance

Inference ran comfortably within the two-second window cycle throughout all test sessions, with no dropped frames or processing delays observed. The display updated smoothly in real time, and BLE transmission to a paired laptop was stable throughout, with results appearing consistently every second as intended.

Battery life across full test sessions comfortably covered a duration suitable for a typical training session or workday monitoring period. Current draw during active inference and BLE transmission stayed within a range that makes all-day wearable use practical on the onboard Li-Po cell, without needing any aggressive power optimization beyond what was already implemented.

The quantized model fit well within the microcontroller's flash capacity, leaving room for future additions to the firmware without requiring hardware changes.

[Insert Fig. 3 — IMU signal pattern showing normal running vs fatigue-onset running]

[Insert Fig. 4 — Assembled prototype worn on wrist during testing]

Discussion

What stood out most across the testing sessions was how much the self-collected dataset contributed to the system working well on the actual hardware. Because the model was trained on data from the same sensors, on the same person, doing the same activities, it did not have to generalize across hardware or population differences. It simply learned the patterns that this specific device produces for this specific person — and that made the outputs noticeably more consistent than we might have achieved with a borrowed dataset.

The fatigue alert is the feature that has the most immediate practical value. Athletes and coaches often make training decisions based on perceived effort, which is subjective and easy to misjudge. Having the device flag fatigue independently — based on changes in movement quality rather than just heart rate or time elapsed — adds an objective data point to that decision. During our testing the alert was never triggered too early or ignored when fatigue was clearly present, which gave us confidence that the underlying signal features are genuinely informative.

The glucose trend component is more of a directional indicator than a precise measurement, and we want to be clear about that. It tells the user whether their metabolic state appears to be shifting in a concerning direction, not what their exact blood glucose number is. For the purpose of general health awareness and encouraging timely medical follow-up, that level of information is still genuinely useful. Precision glucose estimation from a wrist-worn optical sensor remains an open research problem, and this system makes no claim to have solved it — only to have made a meaningful step toward practical, accessible metabolic awareness.

- Conclusion

This project set out to build something practical — a wearable that could monitor glucose trends and physical performance without drawing blood or depending on a server somewhere. We built the hardware, collected the data ourselves, trained the model, compressed it to fit on a microcontroller, and deployed it. The system runs in real time, lasts a full day on battery, and produces results that are consistent and interpretable.

The self-collected dataset turned out to be one of the better decisions we made. Training on data from the actual hardware, from a real person doing the actual activities the device is meant to monitor, produced a model that behaves well in practice in a way that borrowed datasets might not have. The 89.4% accuracy figure is solid for a 24-feature, 11 KB model running on a microcontroller.

Going forward, the natural next steps are expanding data collection to multiple subjects, adding PPG-based heart rate and SpO₂ sensing, and building a proper mobile application for trend logging. The foundation is solid, and there is a clear path to making this genuinely useful for a wider population.

References

- Z [1] International Diabetes Federation, IDF Diabetes Atlas, 10th ed., Brussels, Belgium: IDF, 2021.

- [2] P. Warden and D. Situnayake, TinyML: Machine Learning with TensorFlow Lite on Arduino and Ultra-Low-Power Microcontrollers. O'Reilly Media, 2020.

- [3] P.-A. Lin et al., "Non-invasive blood glucose sensing using near-infrared spectroscopy," Journal of Diabetes Science and Technology, vol. 14, no. 3, pp. 501–512, 2020.

- [4] S. K. Vashist, "Non-invasive glucose monitoring technology in diabetes management: A review," Analytica Chimica Acta, vol. 750, pp. 16–27, 2012.

- [5] M. Stisen et al., "Smart devices are different: Assessing and mitigating mobile sensing heterogeneity for activity recognition," in Proc. ACM SenSys, 2015, pp. 127–140.

- [6] T. Plötz and N. Y. Guan, "Deep learning for human activity recognition in mobile computing," Computer, vol. 51, no. 5, pp. 11–14, 2018.

- [7] A. J. Bandodkar and J. Wang, "Non-invasive wearable electrochemical sensors: A review," Trends in Biotechnology, vol. 32, no. 7, pp. 363–371, 2014.

- [8] TensorFlow Team, TensorFlow Lite for Microcontrollers, 2023. [Online]. Available: https://www.tensorflow.org/lite/microcontrollers

- [9] Arduino, Nano 33 BLE Sense Rev2 — Official Documentation, 2023. [Online]. Available: https://docs.arduino.cc